Chipsatz intern, Intel Triton, Mercury, Neptun , c't 10/95 S.332 (Andreas Stiller)

Translation by Thomas Pabst, Jan 22, 1997

**********************************************************************************

ctchipz.exe  - executable program (current version 3.4, 9/9/96)

intelpci.cfg - Configuration File for Intel PCIsets Neptun, Mercury, Triton FX, HX, VX, Natoma

ctchipz lets you read out and change the chipset configuration, defined by the BIOS.

The correct chipset config registers are preset via a configurration file (ending .cfg).

ctchipz 3.x is able to display and change 8, 16 and 32 bit registers.

Interactive Mode

*********************

For reading and changing an Intel PCIset you simply type

ctchipz intelpci

Alternatively the name and path of the config file can be set as environment variable,

e.g.

set chipset=C:\chipset\intelpci.cfg

in this case you simply type

ctchipz

ctchipz displays the actual settings of the chipset registers.

It stops after each register, so that the user can change the register

contents, quit or move on to the next register.

You can make the following inputs:

S  : Toggles display between

*actual: displaying only the actual setting

*all: displaying all possible settings, where the

actual setting is displayed in red.

M  : Toggles display between

*bitmap: mark interesting bits with 1, other with 0,

e.g. 11100001 for bit 7,6,5,1

*bit list: explicit listing of all bits (default)

e.g. 7..5,2 for bit 7,6,5,1

R  : Repeat last register

I,/: go to new register index (in hex)

?  : simple Help

Q,ESC: quit

CR : or all other not listed keys - next register index

#### Change Registers #####

H,$: Outputs actual register content in hex

this value can be edited, CR-change contents ESC-cancel input

B,%: Outputs actual register content binary

this value can be edited, CR-change contents ESC-cancel input

P  : expects no. of bit in bit pattern and the value you want to change it to

*for 8 bit registers e.g.

7..5,1:=%1111

*for 16 and 31 bit registers you'll have to use two digit numbers, e.g.

07..05,01:=%1111

*in case you only want to change one bit, you type e.g.

P31:=1

###################### W A R N I N G   ###########################

!!!! Changing of some registers can crash or hang your system !!!!

##################################################################

In case you want to start with a particular register, you can enter the index

at the command line, e.g.

ctchipz intelpci /72

command line, batch use, macros

*************************************

You can change registers directly via the command line, e.g.

ctchipz intelpci /5E:=77

changes 8 bit register 5Eh to 77h (if the index is greater than

9Fh, you'll have to add a zero in front of the number, e.g. 0A1h)

The program displays the changed registers and terminates without

waiting for a user input.

alternatively you can enter binary values, e.g.

ctchipz intelpci /5E:=%01110111

you also can enter 'don't care bits' to only change some bits of

the register, leaving the other bits untouched, e.g.

ctchipz intelpci /5E:=%xxxx0010

the number of bits has to match the size of the register (8, 16 or 32 bit)

to make work a little easier, some macros have been pre-defined in the

configuration file. E.g.

ctchipz intelpci /SE0=WON

(changes area E0000..E3FFF to 'write only not cacheable')

in intelpci there are macros in 16kB steps from

/SC0  (C0000..C3FFF) to /SF0  (F0000..FFFFF)

with the following attributes:

rwc   = x111     ;; read write Cacheable

rwn   = x011     ;; read write not Cacheable

won   = x010     ;; Write only not Cacheable

roc   = x101     ;; Read only, cacheable

ron   = x001     ;; Read only not Cacheable

bus   = x000     ;; Read/Write to PCI Bus

There are more macros defined to switch on/off the caches (L1/L2) or

changing to different cache strategies while the system is running.

Please note that you should be careful in virtual mode (Windows, EMM386,

QEMM, 386MAX, OS/2), because the the consistency of data is not guarantied

100%. The secured CPU instruction 'WBINVD' to flush the cache is not available

in virtual mode also. To 'clean' the cache the program needs a memory block of

double the size of the cache. In case of cache sizes of 512 kB or more this can't

be achieved in real mode.

With the command line switch /L2, /L2=512 or /L2=1024, ctchipz is using the protected

mode for cleaning the cache.

You can use several register values, macros and switches in the command line, e.g.

ctchipz intelpci /L2 /58=%xxxxxxx0 /sE0=rwc /sE4=rwc ... /Q

Switch /Q (for 'quiet') only returns error messages, the normal output is not shown.

For larger configurations you can with

/dat=name

use a text file instead of command line parameters. In this text file, the

different register values, macros or files are all starting in a new line.

ctchipz intelpci /dat=my.cfg

my.cfg:

QUIET          ; Switch /Q

L2=512         ; only for 512-K-Caches

58=%xxxxxxx0

sE0=rwc

sE4=rwc

...

to see which macros are defined, please have a look into the configuration files.

You'll usually find L2ON, L2OFF, L1ON, L1OFF, L2WT, L2WB, Dirty etc. The Triton

chipset doesn't know L2ON, you'll need to know the installed L2 cache size and use

L256ON oder L512ON. You can define your own macros as well of course.

Hardware experts can easily write their own configuration files for their chipset,

as long as they have the regarding chipset documentation.